# AI4DI documentation vision group

# Overview

The vision part of the robot takes camera and LiDAR data in order to ascertain the presence of obstacles around the robot (or in front of the robot in testing). An independent system of detections in 2D and 3D was developed, and the independent detections are merged to create 3D detections with information about the position, orientation and class of the obstacle. The system is aware of the robot’s position on the map and is able to filter detected obstacles in zones with known obstacles.

The basic system information:

- 14 packages (10 in diagram, 2 YOLOv3, 2 utilities for saved data)

- ROS2-FOXY

- GIT: https://github.com/LukasKratochvila/ros2-ws-ai4di

- three devices: 2 Raspberry Pi, 1 Jetson nano

An overview of the pipeline as realized

# Arrangement (HW)

The current HW configuration is as follows:

Raspberry Pi 4B RPi - cam

Raspberry Pi 4B RPi - LiDAR

Jetson nano

Raspberry Pi HQ Camera

Livox Converter 2.0

CSI

Livox MID-70 LiDAR

Arducam 1/2″ 2,8 - 12mm f1-1,6 lens

C-mount

M12 aviation

Ethernet

The robot (ROS2)

Ethernet

Ethernet

Ethernet

There are three main threads in the diagram (only logical threads, nothing to do with computational threads):

- camera thread - The camera + lens feeds raw image data into the RPi - cam Raspberry Pi.

- LiDAR thread - feeds raw pointcloud data from the MID-70 into the RPi - LiDAR Raspberry Pi

- image processing thread - the Jetson nano performs the more challenging image processing operations, and is not connected directly to the camera to save computational power purely for the neural network inference

The computational devices consist of two Raspberry Pi 4B units, and a Jetson nano unit. The SW is split among these units in order to use the best performance for each unit, and to ensure that time-critical operations are performed without delays:

- RPi - LiDAR (in green) - handles the LiDAR configuration and readout via the livox_ros2_driver node, and then performs the preprocessing (pcl_preprocessing_node) and clustering (clustering_node) operations

- RPi - cam (in red) - performs camera reading operations (cam_image_node), and then performs some auxiliary operations (such as projecting obstacles detected in the 3D data into the image plane to be matched)

- Jetson nano - performs the neural network inference. The performance of this unit is limited, and ideally it should be replaced with the more powerful Jetson models to achieve better performance, and to be able to handle more image sources.

The diagram below demonstrated the split of tasks between the HW units. The nodes in the green field are handled by the RPi - LiDAR, the nodes in the red field by the RPi - cam. The detector_node is handled by the Jetson nano. The detection_matcher_node is currently run externally, but could be run on the more powerful Jetson if it becomes available. The remaining unassigned nodes are also run externally.

## Raspberry Pi

Both the RPi-LiDAR and RPi-cam are Raspberry Pi 4B 4GB variants with added power supply and cooling boards. Both are running the Ubuntu Mate 20.04 OS. Each unit has installed a ROS2-Foxy environment with a specific workspace as mentioned before. For the RPi-LiDAR it is necessary to install LivoxSDK.

## Jetson nano

The Jetson nano also has 4GB RAM, but beside the CPU: Quad-core ARM Cortex-A57, has GPU: NVIDIA Maxwell architecture with 128 NVIDIA CUDA® cores. Jetson running on JetPack SDK4.6.1 from NVIDIA. It is able to run even neural networks at sufficient speed. We use docker image from GIT: https://github.com/dusty-nv/jetson-containers. In this container is installed a ROS2-Foxy environment with a specific workspace.

# Packages (SW)

The following packages have been created:

Below the packages are described as far as their interface (inputs and outputs), internal workings, and algorithms used (in alphabetical order, for the pipeline arrangement see the images above):

## clustering

- IN: preprocessed point cloud sensor_msgs/msg/PointCloud2

- OUT: clusters vision_msgs/msg/Detection3DArray

Node takes the preprocessed point cloud and finds clusters using the Euclidean clustering method. The found clusters (if any) are represented as bounding boxes. Either AABB (axis-aligned bounding boxes), or OBB (oriented bounding boxes) can be selected. The node operates as follows:

- Receive preprocessed point cloud msgs =>

- check content (if empty republish empty msg) =>

- use pcl::search::KdTree and pcl::EuclideanClusterExtraction =>

- find clusters (if none found pushlish empty msg) =>

- find AABB or OBB with inner function (OBB tries find the AABB with minimum footprint by rotating the cluster point cloud, mode and angle increments are set in the launch parameters) =>

- publish clusters msg

| clustering_node: ros__parameters: cluster_tolerance: 0.2 #in meters min_cluster_size: 30 max_cluster_size: 5000 input_topic: "/pcl_preprocessing_node/filteredPcl" output_topic: "~/detections_clusters" OBB: true step: 30 debug: false | Clustering node parameters How far apart points are still one cluster (in meters) Minimum cluster size (in points) Maximum cluster size (in points) Input topic identifier Output topic identifier Bounding box mode switch (true=OBB, false=AABB) Angle step for OBB search Debug switch (true for verbose execution) |
| --- | --- |

## darknet_vendor (non-original)

- Taken over from: https://github.com/ros2/darknet_vendor

- CMake wrapper around darknet, an open source neural network framework.

## detection_matcher_py

- IN: clusters vision_msgs/msg/Detection3DArray and  2Ddetections, 2Dprojections both vision_msgs/msg/Detection2DArray

- OUT: matched detections vision_msgs/msg/Detection3DArray

Node takes 2D detection (from YOLO) and 2D projection (from Clusters) and tries to match to 3D detections (clusters) for label transfer (from 2D YOLO detection to 3D cluster detection). The result then should be groups of corresponding 2D and 3D detections, published as 3D detections with the associated class.

- Receive 3 approximate synchronize msgs clusters, 2Ddetections, 2Dprojections =>

- control sizes of projection and 3D detection (should be same) =>

- one by one match 3D detection with the highest IOU (Intersection over Union) candidate and associate 3D detection with its label  =>

- publish matched detections msg

| detection_matcher_node: ros__parameters: Project2D_topic: "/projection_node/projections" Detect2D_topic: "/detector_node/detections" Detect3D_topic: "/clustering_node/detections_clusters" Output3D_topic: "~/matched_detections" queue_size: 100 time_toll: 0.1 tresh: 0.5 debug: false | detection_matcher_node parameters Project2D topic identifier Detect2D topic identifier Detect3D topic identifier Output topic identifier Queue size for message synchronization Time toll parameter for message synchronization Threshold parameter for IoU metric Debug switch (true for verbose execution) |
| --- | --- |

## detection_vizualizer

Part taken over from: https://github.com/ros2/detection_visualizer

Package for visualization 2D detections (from YOLO), 3D detections and image decompression.

### DetectionVisualizerNode

- Node for visualization 2D detection.

- IN: image sensor_msgs/msg/Image and 2Ddetections vision_msgs/msg/Detection2DArray

- OUT: image with detections sensor_msgs/msg/Image

### Det3dVisualizerNode

- Node for visualization 3D detection. Detection 3D to MarkerArray converter.

- IN: 3Ddetection vision_msgs/msg/Detection3DArray

- OUT: 3Ddetection visualization_msgs/msg/MarkerArray

### Decompressor

- Node for decompression of compressed images. Compressed Image to Image converter.

- IN: compressed image sensor_msgs/msg/CompressedImage

- OUT: decompressed image sensor_msgs/msg/Image

Only for debugging and presentation.

## image_tools_custom (non-original)

Taken over from: https://github.com/ros2/demos/tree/foxy/image_tools. Edited node for image publishing. (Added properties, see the param file example)

- OUT: raw image sensor_msgs/msg/Image and compressed image sensor_msgs/msg/CompressedImage

| cam_image_node: ros__parameters: reliability: 'reliable' history: 'keep_last' depth: 10 frequency: 30.0 burger_mode: false show_camera: false width: 640 height: 480 frame_id: image output_topic: '/detector_node/images' device_id: 0 api_id: 0 cap_mode: 0 debug: false | image_tools_custom node parameters Reliability QoS setting. Either 'reliable' (default) or 'best_effort' History QoS setting. Either 'keep_last' (default) or 'keep_all'. If 'keep_last', then up to N samples are stored where N is the depth Depth of the publisher queue. Only honored if history QoS is 'keep_last'. Default value is 10 Publish frequency in Hz. Default value is 30 From the original node - unused From the original node - unused Width component of the camera stream resolution. Default is 320 Height component of the camera stream resolution. Default is 240 ID of the sensor frame. Default value is 'camera_frame' Output ROS topic. Default value is 'image' ID of the camera source. Default value is '0' ID of the opencv api. Default value is '0' Capture mode (only for libv4l). Default value is '0' Debug switch (true for verbose execution) |
| --- | --- |

## img_pub_py (utility)

Node for publishing saved images from folder at a given frequency.

- OUT: images sensor_msgs/msg/Image

## livox_ros2_driver (non-original)

Taken over from: https://github.com/Livox-SDK/livox_ros2_driver. Livox device driver under Ros2, support Lidar Mid-40, Mid-70, Tele-15, Horizon, Avia. Node for reading and publishing raw point cloud from Livox sensor. Enables configure measurements parameters.

- OUT: raw point cloud sensor_msgs/msg/PointCloud2

| livox_node: ros__parameters: xfer_format: 0 multi_topic: 0 data_src: 0 publish_freq: 10.0 output_data_type: 0 frame_id: "cloud" user_config_path: "path_default" cmdline_input_bd_code: "000000000000001" lvx_file_path: "/home/livox/livox_test.lvx" | livox_ros2_driver node parameters 0-Pointcloud2(PointXYZRTL), 1-customized pointcloud format 0-All LiDARs share the same topic, 1-One LiDAR one topic 0-lidar,1-hub frequency of publishing,1.0,2.0,5.0,10.0,etc freq from (0.1, 100) Output to ros: 0, Output to bagfile: = 1 Output frame id User config file path Default BD code LVX file path |
| --- | --- |

## map_lookup

- IN: candidate detections visualization_msgs/msg/MarkerArray

- OUT: new detections nav_msgs/msg/OccupancyGrid

The node takes detection candidates from the detection pipeline, and looks in the map whether the detection falls into an area of known obstacles, or whether this is actually an unknown obstacle. The algorithm for each candidate is as follows:

- transform the candidate detection into map coordinates =>

- generate a grid (5x5 by default) of points describing the footprint of the bounding box =>

- check the occupancy grid in all the points (25  by default) (the box might not be entirely in the occupied/clear area), and increase a counter for each point in the occupied area =>

- compare the accumulator to a threshold (threshold by default 0.2) =>

- publish new detections, which satisfy the threshold condition

The node is not currently used in the pipeline, instead pcl_preprocessing map crop feature is used. If map_lookup is to be used, changes are needed.

| map_lookup_node: ros__parameters: input_topic: "/clustering_node/detections" output_boxes_topic: "~/unknownObstacles" output_map_topic: "~/ObstacleMap" synchronised: false map_server: "/map_server/map" debug: false | map_lookup_node parameters Input topic identifier Output box topic identifier Output map topic identifier Synchronisation switch (default false) Map topic identifier Debug switch (true for verbose execution) |
| --- | --- |

## openrobotics_darknet_ros (non-original)

This is a ROS 2 wrapper around darknet, an open source neural network framework.

- Taken over from: https://github.com/ros2/openrobotics_darknet_ros

- IN: image sensor_msgs/msg/Image

- OUT: 2Ddetections vision_msgs/msg/Detection2DArray

The node runs the selected model (in this case YOLOv3) on any received image message, and returns the detections in form of a 2D detection message, including the position, size, class and confidence of the detection.

The CNN model is loaded from a file and inference is performed according to the model. Training is performed externally either as a retraining of an already trained model on a given dataset, or from scratch on a given dataset.

| detector_node: ros__parameters: network: config: "./model/yolov3-tiny.cfg" weights: "./model/yolov3-tiny.weights" class_names: "./model/coco.names" detection: threshold: 0.25 nms_threshold: 0.45 | map_lookup_node parameters Config file path Model weights file path Class name legend file path Detection confidence threshold (above is accepted, below rejected) Non Maximum Supression threshold (to weed out overdetections) |
| --- | --- |

## pcl_preprocessing

- IN: raw input cloud sensor_msgs/msg/PointCloud2

- OUT: filtered cloud sensor_msgs/msg/PointCloud2

- OUT (optional): local map nav_msgs/msg/OccupancyGrid

Node reads raw pointcloud data provided by the livox_ros2_driver node, and performs preprocessing necessary for further operations such as clustering. The work flow is as described below:

cloud crop

detect floor

downsample

map crop

raw LiDAR

cloud output

map

local map

sensor_msgs/msg/PointCloud2

sensor_msgs/msg/PointCloud2

nav_msgs/msg/OccupancyGrid

Upon receiving the pointcloud message, following operations are performed (particular configuration of the node depends on the parameters at launch:

- cloud crop - crops the raw cloud by a fixed (non-smart) crop box (dimensions specified in the parameters at launch). This removes unwanted points - too far ahead, to the side, above and below. The purpose is to reduce the cloud size for performance

- detect floor - Detects the largest near horizontal surface that is described by the cloud, then removes the points representing that surface

- downsample - downsampled the cloud using the voxelgrid method. This removes a significant amount of points reducing computational power requirements, it also reduces the point cloud density in areas close to the sensor, and retains density in less dense areas

- map crop - removes points in places of already known obstacles (from the map). Obstacle detections are then not generated in areas already marked as obstacles

- frame aggregator - optionally (not currently used) the frame aggregator concatenates pointcloud frames transformed by the corresponding position and orientation of the sensor in order to generate a dense point cloud - the aggregation has problems with faster moving objects if the aggregation periods are longer

A filtered cloud is then output as a basis for further processing. Optionally a local map occupancy grid is also published.

| pcl_preprocessing_node: ros__parameters: crop: true detectFloor: false downSample: true mapCrop: true enableAgg: false createLocalMap: false input_topic: "/livox/lidar" output_cloud_topic: "~/filteredPcl" output_map_topic: "~/localMap" filter_x_min: 0.1 filter_y_min: -10.0 filter_z_min: -0.2 filter_x_max: 25.0 filter_y_max: 10.0 filter_z_max: 2.0 voxel_grid_x_res: 0.025 voxel_grid_y_res: 0.025 voxel_grid_z_res: 0.025 mapTreshold: 200 map_frame: "map" agg_window: 2e8 costmap_node_name: "pcl_preprocessing_costmap" local_costmap_namespace: "pcl_preprocessing_costmap" debug: false | pcl_preprocessing_node parameters Cloud crop switch Floor detect switch Downsample switch Map crop switch Frame aggregator switch Local map switch Input topic identifier Output cloud topic identifier Output map topic identifier Crop parameters (XYZ in Livox MID-70 frame, all in meters) Voxel Grid resolution in X and Y, keep them equal Voxelgrid resolution in Z Map threshold for occupied / clear Map frame identifier Frame aggregator time span in ns Costmap identifier Local map identifier Debug switch (true for verbose execution) |
| --- | --- |

An example of the pcl_preprocessing.

## pcl_pub_cpp (utility)

Node for publishing saved point clouds (from .pcd files) at a given frequency.

- OUT: point cloud sensor_msgs/msg/PointCloud2

## projection

- IN: clusters visualization_msgs/msg/MarkerArray

- OUT: 2Dprojections vision_msgs/msg/Detection2DArray

- OUT (optional): img_publisher sensor_msgs/msg/Image

- OUT (optional): pcl_publisher visualization_msgs/msg/MarkerArray

Node takes clusters and projects them to camera space (image coordinates) using a known transformation. The projection allows for mapping of 3D detections onto the 2D detections. This is an inverse perspective projection problem which requires camera calibrations (or more precisely intrinsic camera parameters), the camera was calibrated externally and intrinsic parameter matrix is used as parameter. Any camera change, lens change (or zoom change) requires a camera calibration. The workflow is as follows:

- Receive clusters msg =>

- for each cluster try to transform marker to image frame =>

- project front side and center of 3D object to 2D => (here problems may be encountered using the OBB representation, the selected face may not be the best representation (requires some changes to code))

- publish 2Dprojections msg

| projection_node: ros__parameters: input_topic: "/clustering_node/detections" output_topic: "~/projections" output_pcl_topic: "~/pcl" output_img_topic: "~/img" output_frame: "image" debug: false | map_lookup_node parameters Input topic identifier Output projections topic identifier Output cloud topic identifier Output image topic identifier Output topic frame identifier Debug switch (true for verbose execution) |
| --- | --- |

## tracker

- IN: detections vision_msgs/msg/Detection3DArray or vision_msgs/msg/Detection2DArray

- OUT: tracked detections vision_msgs/msg/Detection3DArray or vision_msgs/msg/Detection2DArray

- Part taken over from: https://github.com/nwojke/deep_sort

Node realizes tracking of already detected potential obstacles. It uses the DeepSORT method to achieve tracking using Kalman filtration, and deep association between instances using neural networks. Kalman filtration predicts position of tracked object and deep association uses a deep appearance descriptor (only works with images in this implementation). Would work better with higher detection framerate.

Node works in three modes: 3D or 2D tracker and the 2D tracker with or without deep features. It takes detections, converts them to deep_sort format and tracks them with a kalman filter. The workflow is as follows:

- Receive detections msg =>

- if deep features are enabled find it =>

- convert detections into deep_sort format=>

- predict new positions (use deep_sort) =>

- update positions =>

- convert detections to vision_msgs format  =>

- publish tracked detections msg

In 2D mode

| tracker_2d_node: ros__parameters: input_det_topic: '/detector_node/detections' output_det_topic: '~/detections_tracker' dim: 2 model_path: "model/tracker-mars-small128.pb" metric: "euclidean" matching_threshold: 0.2 budget: 10 max_iou_distance: 0.9 max_age: 30 n_init: 3 debug: false | tracker_2d_node parameters Input topic identifier Output detection topic identifier Dimensions to track (2D in this case) Path to the DeepSORT model Distance metric Threshold of matching Budget - how many latest samples to remember How many frames can a sample be old Debug switch for verbose execution |
| --- | --- |

In 3D mode

| tracker_3d_node: ros__parameters: input_det_topic: '/clustering_node/detections_clusters' output_det_topic: '~/detections_tracker' dim: 3 model_path: None metric: "euclidean" matching_threshold: 0.2 budget: 10 max_iou_distance: 0.9 max_age: 3 n_init: 3 debug: false | tracker_3d_node parameters Input topic identifier Output detection topic identifier Dimensions to track (3D in this case) Path to the DeepSORT model Distance metric Threshold of matching Budget - how many latest samples to remember How many frames can a sample be old Debug switch for verbose execution |
| --- | --- |

## writer_cpp (utility)

Node for saving images and/or point clouds into .pcd files. Merely a utility.

- IN: image sensor_msgs/msg/Image

- IN: point cloud sensor_msgs/msg/PointCloud2

| writer_node: ros__parameters: save_dir: "./" img_sub_dir: "image/compressed" pcl_sub_dir: "pcl/" input_img_topic: "/image" input_pcl_topic: "/livox/lidar" mode: "both" synchronized: false time_toll: 0.01 start_img_counter: 0 start_pcl_counter: 0 img_file_ext: "jpg" pcl_file_ext: "pcd" debug: true | writer_node parameters Save directory path Image subdirectory name Clout subdirectory name Image input topic name Cloud input topic name Save mode: “img”, “pcl” or “both” Synchronised switch Time toll for synchronization Filename counter start for image Filename counter start for cloud Image file extension Cloud file extension Debug switch for verbose execution |
| --- | --- |

# Startup

The launch of the perception pipeline is realized via ROS2 launch files (particular launch file examples for this project). For each hardware unit a custom launch file exists launching the particular nodes that are to be run on said unit. A common params file exists for all the units (an example for this project), excerpts from which are shown above. This params file dictates the configuration of all the nodes including their parameters.

Currently the HW is set up for automatic launch of all nodes in the pipeline.

# Datasets

During testing we collected 3 different datasets. The first one was in an industrial environment with different situations such as planning way, obstacle detours and emergency stopping. This one is the closest to the real situation. Unfortunately it contains several errors.  Second one was developed in the laboratory at the University. In the lab there are many different objects.  The last dataset was created in a small room testbed on CEITEC. It is the most complete dataset. Only small error is bad calibration of map origin.

# Visualization

## Lidar preprocessing pipeline

Raw point cloud

Associated Image

Floor detected and removed

Cropped by a fixed box (extra points too far removed)

Cropped by a fixed box and downsampled

## Clustering

## Projection

3D detections projected into the image (note the missing class information and confidence, but the present distance from the robot (assumed from the LiDAR sensor origin)

## YOLO detection

Visualized YOLO detection

## Matcher

Visualization of the 3D detection from matcher node with associated label and probabilities (label: person, confidence: 1.0)